《Diecutting Machine BD-1050C》

》Diecutting Machine BD-1050C >> Machinery Dimension

》Diecutting Machine BD-1050C >> Main Functions

- LCD serene dialogue window displays all kinds of information with recording and setting functions. - Feeder can start to feed the first sheet of paper during high speed operation with fast speed and high accuracy. - Adopts paper catch pincer that can ensure accuracy during high speed operation. - Electronic control for slight left and right adjustment of feeder platform. - Paper feed belt buffer structure. Thin paper can be die cut easily, quickly, and accurately. - Lubrication oil constant temperature control that can solve the unevenness of cold shrinkage and heat expansion problem and can also protect machine from wear and tear. - Electronic control digital display for die cut pressure. Operation is easy, clear and fast. - Imported excellent hardness die cut steel plate. Solid and durable and is hard to be damaged. - Die cut pressure protection device. Automatic shutdown when pressure is over 300 tons so as to ensure mechanical safety. - Electronic control air pressure type for wedging of die frame. Speedy, light and easy operation. - European main chain and vacuum pump. Durable and not easy to be damaged. - Comes with non-stop feeding and delivery with automatic subsidiary paper collection rack. Speedy automatic conversion.

| 》Diecutting Machine BD-1050C >> Machinery Specifications |  |
| --- | --- |
| max. paper size | 1050mm × 750mm |
| min. paper size | 400mm × 370mm |
| max. die cutting size | 1040mm × 730mm |
| max. cutting pressure | 300tons |
| min./max/ thickness paper | 80g/㎡ ~ 1500g/㎡ |
| max/ corrugated board | 4mm |
| min. gipper margin | 8mm |
| max. machine speed | 7500 Sheets/hr |
| inner size of chase | 1145mm |
| size of steel plate | 1080mm × 736mm |
| max.feeder overlap height （with platform） | 1250mm |
| max. delivery overlap height （with platform） | 1150mm |
| machine total power | 22kw |
| machine length | 6110mm |
| machine width | 3470mm |
| machine heihth | 2500mm |
| total weight(approx.) | 14.3 tons |
| ★ Specifications are change without notice for improvement. |  |

》Diecutting Machine BD-1050C >> Machinery Introduction

Touch-screen

- Display all information during operation. - Setting function for die cutting pressure/paper insert length indications etc.

Paper feeding divice

- Excellent and delicate feeder design that allows steady and accurate paper feeding - Height and angle of one suction and two suctions can be adjusted individually - Height and angle of first suction and second suctions can be adjusted individually. - Outfitted with preparation paper accumulation platform to enhance production volume and performance. - Outfitted with non-stop feeding speedy automatic conversion between main/ auxiliary paper rack.

Conveyance positioning structure

- Push/pull device for horizontal positioning.Conversion is easy and simple and facilitate sselection of paper. - During operation the two sets or front positioning can arbitrarily adjust the tolerance of front and back/slanting. - Double sheet device that can check whether there is double sheet feeding and it will shutdown to protect machine. - Automatic shutdown device that can check for abnormal precision of paper feed front / horizontal positioning. - Paper feed belt buffer device – to stabilize accuracy of thin paper.

Pressing unit

- US import pressure controller.Automatic shutdown when die cut pressure is over 300 tons in order to ensure mechanical safety. - Electrical equipment digital control for die cut pressure adjustment.Can be displayed on the screen for recording and setting. - Die frame adopts air pressure pinching.

Die cutting unit

- Curve operation structure with heavy loading durability Stable pressure, high precision,wear and tear durability. - Safety die cutting frame design, Easy installation and dismantling. Rotation die frame design that allows easy replacement of die cutters. - Imported excellent hardness die cut steel sheet,durable and wear and tear resistance.

Delivery unit

- Brush and lower/ side nozzle are utilized to stabilize effect of uniform paper collection. - No-stop delivery for full paper during operation. - To set up paper indicator insert device - Can set total paper number and automatic shutdown function

寶德企業股份有限公司 　　Baoder　Enterprise　Co.,Ltd.

NO.178-7,Wen-cheng Rd. Taishan,Taishan,Taipei County,Taiwan

TEL：886-2-2900-5551~53　FAX：886-2-2900-8633